ARDUINO BEGINNERS COURSE Lesson 3: Arduino UNO Complete + Introduction to the Breadboard Student Notes ¦ Ages 10–15 ¦ Safety First!

| LESSON INFO | DETAILS |
| --- | --- |
| Lesson | 3 — Arduino UNO Complete + Breadboard Introduction |
| Builds On | Lessons 1 & 2: Tinkercad interface, basic and detailed UNO parts |
| Key Focus | Remaining UNO parts, breadboard anatomy, safe connections |
| Safety Note | ⚠️ Understanding before connecting — prevents short circuits! |

SECTION 1 ¦ REMAINING ARDUINO UNO PARTS — COMPLETING THE PICTURE

In Lessons 1 and 2 we covered the main parts of the Arduino UNO. In this section we complete the picture by looking at the remaining parts you will encounter as you progress.

## 1.1  Power LED (ON)

There is a small green LED on the Arduino board labelled 'ON'. This lights up whenever the board has power — either from USB or from the barrel jack. If this LED is NOT on, the board is not receiving power.

💡 Remember: If your ON LED is not lit, check your USB connection or power supply before doing anything else. Never assume the board is working if this LED is off.

## 1.2  PWM Pins — Pulse Width Modulation (Deep Dive)

We introduced PWM pins in Lesson 2. Now let's understand them properly because you WILL use them in projects.

PWM stands for Pulse Width Modulation. Instead of truly producing varying voltages (like 2.5V), a PWM pin switches ON and OFF very rapidly. The ratio of ON time to OFF time is called the DUTY CYCLE.

| PWM CONCEPT | EXPLANATION |
| --- | --- |
| PWM Pins | Pins 3, 5, 6, 9, 10, 11 (marked with ~) |
| Duty Cycle 0% | Pin stays OFF — LED is fully off |
| Duty Cycle 50% | Pin switches on/off equally — LED appears half bright |
| Duty Cycle 100% | Pin stays ON — LED is fully bright |
| Code Command | analogWrite(pin, value) — value is 0 to 255 |
| Example | analogWrite(9, 127) — sets pin 9 to ~50% brightness |

💡 Real Life Comparison: Think of PWM like a light switch you flick ON and OFF 1000 times per second. If you flick it ON half the time and OFF half the time, the light LOOKS half as bright — even though it is only ever fully on or fully off!

## 1.3  Serial Communication Pins (TX and RX)

| PIN | PURPOSE |
| --- | --- |
| Pin 0 — RX | RECEIVE. The Arduino receives data through this pin from other devices or the computer. |
| Pin 1 — TX | TRANSMIT. The Arduino sends data through this pin to other devices or the computer. |
| Serial Monitor | In Tinkercad and Arduino IDE, you can use Serial.println() to send messages to your computer screen — great for debugging! |
| IMPORTANT | Avoid using pins 0 and 1 for other purposes when using USB communication — they share the same line. |

## 1.4  ICSP Headers (In-Circuit Serial Programming)

You will see two sets of 6 pins on the Arduino board — one near the ATmega328P and one near the ATmega16U2. These are ICSP headers.

- They are used to program the chips directly — bypassing the USB bootloader.

- This is used by advanced users and manufacturers — not needed for beginner projects.

- NEVER connect anything to ICSP unless you know exactly what you are doing.

⚠️ Safety Rule: Leave the ICSP headers alone for now. They look like regular pins but connecting them incorrectly can permanently damage the microcontroller chip.

## 1.5  IOREF and AREF Pins

| PIN | PURPOSE |
| --- | --- |
| IOREF | Tells external shields what voltage the Arduino operates at (5V for UNO). Used by shields to adapt automatically. |
| AREF | Analog Reference. Lets you set a custom reference voltage for analog readings instead of the default 5V. Advanced use only. |

📌 For beginners: You do not need to use IOREF or AREF in your first 10 projects. Leave them unconnected.

SECTION 2 ¦ INTRODUCTION TO THE BREADBOARD

Now that we fully understand the Arduino UNO, we need to understand the BREADBOARD — because the two work together in almost every beginner project.

🔥 Why We Learn This BEFORE Connecting: A short circuit happens when electricity takes an unintended path — usually because components are connected incorrectly on the breadboard. Short circuits can permanently damage your Arduino. Understanding the breadboard completely PREVENTS this.

## 2.1  What is a Breadboard?

A breadboard (also called a solderless breadboard) is a plastic board with hundreds of tiny holes arranged in rows and columns. You push component legs and wires into these holes to build circuits — NO soldering required.

- It is reusable — you can build, test, and rebuild circuits as many times as you want.

- It is safe for beginners — no hot tools or permanent connections.

- It is the standard tool for prototyping (testing ideas before making them permanent).

## 2.2  Anatomy of a Breadboard

This is the MOST IMPORTANT thing to understand. The holes in a breadboard are connected in a very specific pattern INSIDE the board, even though you cannot see the connections.

| PART | HOW IT WORKS |
| --- | --- |
| Terminal Strips (Middle Section) | The main area. Holes are arranged in columns labelled A–E on the left and F–J on the right, and rows numbered 1–30 (or 1–60). Each ROW of 5 holes (A–E or F–J) is connected internally. Inserting two wires in the same row connects them. |
| Centre Divider (The Gap) | A gap runs down the middle of the board. This SEPARATES the left side (A–E) from the right side (F–J) in the same numbered row. This gap is designed to fit IC chips across. |
| Power Rails (Outer Columns) | Two long columns run along each side, marked + (red) and – (blue/black). ALL holes in the + column are connected together. ALL holes in the – column are connected together. These are used to distribute power (5V) and ground (GND) across the board. |

💡 Simple Rule to Remember: Holes in the SAME ROW (same number, same side) are connected. Holes in the SAME COLUMN on the power rail are connected. Everything else is NOT connected — even if holes look close together.

## 2.3  The Power Rails Explained

The power rails run along the long sides of the breadboard. They are the first thing you connect when setting up any circuit:

| RAIL | PURPOSE |
| --- | --- |
| Red Rail ( + ) | Connect to Arduino 5V pin. This distributes 5 volts across the whole rail — any component that needs power connects here. |
| Blue/Black Rail ( – ) | Connect to Arduino GND pin. This is your ground line — every circuit must return to ground through this rail. |
| Why Use Rails? | Instead of running wires from every component directly to the Arduino pins, you connect the rails ONCE and then tap into the rail anywhere on the board. Cleaner and safer. |

## 2.4  How Components Sit on the Breadboard

Most electronic components have wire legs called leads. You push these leads into the breadboard holes:

- Each lead goes into a DIFFERENT row — this connects it to that row's internal connection.

- To connect two components, put one lead from each into the SAME ROW.

- To power a component, connect one lead to the + rail and the other (through a resistor) to the – rail.

⚠️ Critical Safety Rule: NEVER put both legs of a component into the SAME row. This creates a direct short circuit across that component — potentially burning it out or damaging the Arduino.

SECTION 3 ¦ COMMON SHORT CIRCUIT MISTAKES — AND HOW TO AVOID THEM

A short circuit is one of the most common mistakes beginners make. Here are the specific mistakes and how to avoid each one:

| MISTAKE / FIX | EXPLANATION |
| --- | --- |
| ❌ MISTAKE 1 Connecting 5V directly to GND | Without any component in between, connecting the 5V pin directly to the GND pin creates a short circuit. The Arduino will get very hot and may be permanently damaged. |
| ✅ FIX | Always have a component (LED, resistor, motor) between 5V and GND. Never connect them directly. |
| ❌ MISTAKE 2 No resistor with an LED | An LED has almost no resistance. Without a resistor, too much current flows through it. The LED burns out and the Arduino pin may be damaged. |
| ✅ FIX | Always use a 220Ω resistor in series with any LED. Connect: Pin → Resistor → LED → GND. |
| ❌ MISTAKE 3 Two legs in the same breadboard row | If both legs of a component go into the same row, the current bypasses the component — creating a short circuit. |
| ✅ FIX | Each leg of a component must go into a DIFFERENT numbered row on the breadboard. |
| ❌ MISTAKE 4 LED inserted backwards | LEDs only allow current in one direction. If inserted backwards, it will not light up — and if too much voltage is applied, it may burn out. |
| ✅ FIX | The LONG leg (anode) connects toward the positive side. The SHORT leg (cathode) connects toward GND. |
| ❌ MISTAKE 5 Power connected while building | Making changes to your circuit while the Arduino is powered on can cause accidental short circuits when wires touch. |
| ✅ FIX | Always UNPLUG or DISCONNECT power before adding, removing, or changing any wires or components. |

🔑 Golden Safety Rule: BUILD FIRST. POWER SECOND. Always complete your circuit completely and check it carefully BEFORE connecting power. This single habit prevents 90% of beginner mistakes.

SECTION 4 ¦ YOUR FIRST SAFE CIRCUIT — LED ON BREADBOARD

Now you know enough to safely build your first circuit. Here is the correct, safe way to connect an LED using a breadboard and Arduino UNO:

## Components Needed (in Tinkercad):

- 1 × Arduino UNO

- 1 × Breadboard (small or full size)

- 1 × LED (any colour)

- 1 × Resistor — set to 220Ω

- 3 × Jumper Wires (red, black, and one other colour)

## Safe Connection Steps:

| STEP | ACTION |
| --- | --- |
| Step 1 | Place the Arduino UNO and breadboard on the Tinkercad canvas — do NOT connect power yet. |
| Step 2 | Connect Arduino 5V pin → breadboard red (+) power rail using a RED wire. |
| Step 3 | Connect Arduino GND pin → breadboard blue (–) power rail using a BLACK wire. |
| Step 4 | Place the resistor (220Ω) across the breadboard — one leg in row 10, other leg in row 14 (different rows!). |
| Step 5 | Place the LED — long leg (anode) in row 14 (same as resistor end), short leg (cathode) in row 16. |
| Step 6 | Connect a wire from the blue (–) rail to row 16 (LED cathode to GND). |
| Step 7 | Connect a wire from Arduino Pin 13 to row 10 (Arduino to resistor). |
| Step 8 | CHECK: Each component leg is in a DIFFERENT row. Resistor between Pin 13 and LED. LED cathode to GND. |
| Step 9 | NOW connect power and simulate. The LED should light up! |

// Lesson 3: LED on Breadboard via Pin 13 void setup() { pinMode(13, OUTPUT); // Set Pin 13 as output } void loop() { digitalWrite(13, HIGH); // LED ON delay(1000); // Wait 1 second digitalWrite(13, LOW); // LED OFF delay(1000); // Wait 1 second }

SECTION 5 ¦ QUICK REFERENCE — BREADBOARD SAFETY CHECKLIST

Before powering ANY circuit, run through this checklist:

| CHECKLIST ITEM | WHY IT MATTERS |
| --- | --- |
| ☐ Power OFF | Confirm Arduino is not connected to USB or power while building. |
| ☐ LED direction | Long leg toward positive. Short leg toward GND. |
| ☐ Resistor present | Every LED must have a 220Ω resistor in series. |
| ☐ Different rows | Each component leg is in a different numbered row. |
| ☐ Power rails correct | Red wire to 5V → + rail. Black wire to GND → – rail. |
| ☐ No bare wires touching | No exposed wire ends touching each other or the Arduino board. |
| ☐ Double check | Look at your circuit one more time from the beginning. |
| ☐ NOW power on | Only now connect USB or power supply. |

Lesson 3 Complete! You now know every part of the Arduino UNO AND how a breadboard works. Next Lesson: Build your first LED blink circuit on a real breadboard in Tinkercad!